Simulation of Gas Bremsstrahlung Radiation from APS Undulator Straight Sections using MARS

J. Dooling and L. Emery

Argonne National Laboratory, 9700 South Cass Avenue, Argonne, IL 60439, USA

Abstract

Simulations of gas bremsstrahlung (GB) radiation generated in 15.38-m long undulator straight sections of the Advance Photon Source (APS) storage ring (SR) have been carried out using the Monte Carlo particle-matter interaction program MARS.  MARS results are compared with earlier simulations conducted with EGS4, analytical modeling, and direct measurements.  A spatial distribution of GB photons expected to be present with synchrotron x-rays downstream of the straight section source is generated using MARS and compared with a simple analytical model.  Total dose equivalent levels recorded in a phantom placed in contact downstream of heavy-metal beam stops are compared as a function of stop thickness.  Generation of GB photoneutrons is simulated with MARS for several target materials to compare with previous measurements [1].  Differences between measured and simulated results are observed and discussed.  A method to assess the required shielding thickness required between the GB extreme ray and a beam stop edge is described.  Large numbers of events can be simulated with a parallel version of MARS running on an APS Linux cluster.  Data visualization and analysis are facilitated both through the SDDS [2] suite of tools as well as the built-in MARS GUI.  The SDDS toolkit works in conjunction with a Physics Analysis Workstation (PAW) interface to extract binary histogram data for post processing.

References

- M. Pisharody et al., NIM-A, 430, 542 (1999).

- M. Borland, L. Emery, ``The Self-Describing Data Sets File Protocol and Toolkit,'' Proceedings of 
ICALEPS '95, October, 1995, Chicago, Illinois, Fermilab Report CONF-96/069, p. 653 (1996).